## 1. General Car Requirements

- Any V6 or V8 engine is allowed, provided it meets all safety requirements in the general rules.

- Front- or rear-wheel drive vehicles permitted (cars, trucks, vans).

- No 4-wheel drive or all-wheel drive.

- No turbos, superchargers, or nitrous.

- Fuel: Gas only (maximum 110 octane). No E85 or C85.

- Minimum weight:

  - 3200 lbs. with driver

  - 3250 lbs. for three-speed transmission cars

## 2. Body & Frame

- Stock or aftermarket bodies allowed. No slanted roofs or late-model noses.

- Maximum 5-inch spoiler allowed.

- Front firewall must remain stock and in its original location.

- Stock floor pans required from the front firewall to the rear; trunk pan may be removed.

- Rear of car must be fully enclosed.

- A solid firewall is required between the fuel cell/gas tank and driver’s compartment.

  - Stock fuel tanks must remain in the stock location.

- All holes in firewall, floor pans, and rear firewall must be sealed.

- Frame must be full-length; no lightening or shortening.

- Rear frame rails may be replaced with square tubing behind the rear-end only.

## 3. Engine & Transmission

### Engine

- Engine must remain in the stock location (no setback). Aftermarket mounts allowed.

- No mid-plate mounts.

- Max displacements: Chevy 361 cu in (no 400 blocks), Ford 351 cu in.

- Cast-iron intake only — no aluminum, no Bowtie, no high-rise, no porting/polishing.

- Any HEI distributor allowed.

  - No MSD or ignition boxes.

- Cylinder heads:

  - Stock factory cast iron heads only

  - No double-hump or Vortec heads

  - Valves: 1.94 / 1.50 max

  - Stock 1.5 rocker arms with stock markings

  - Screw-in or pinned studs and guide plates allowed

- Exhaust: Stock cast-iron manifolds only. No headers.

- Pushrods must be stock length (no shortening or lengthening).

- Camshaft:

  - Hydraulic only

  - Max .450” lift anywhere on the cam rotation

- Pistons: Flat-top or dish only, max .060" overbore.

- Connecting rods: 5.7" or 6” I-beam only; no lightweight rods.

- No strokers or destrokers. Pistons may not protrude from block.

- Crankshaft:

  - Stock-appearing

  - No lightweight

  - Minimum 50 lbs

- Allowed aftermarket parts: valve covers, air cleaner, power steering pump, racing oil pan, aluminum pulleys, aluminum radiators.

- No aftermarket engine blocks.

- Quick-steer and aftermarket steering shafts allowed.

- No other high-performance parts.

### Transmission

- Stock transmissions only.

- Automatics must use a stock, functioning torque converter.

- Manual transmissions must use stock flywheel, pressure plate, and clutch.

  - No triple-disc clutches

  - No racing clutches or lightened flywheels

- Aftermarket clutch pedal allowed.

- No direct-drive transmissions.

- Fuel-injected or throttle-body engines must have stock internals.

## 4. Carburetor Rules

- Allowed options:

  - Unaltered Quadrajet

  - Gauge-legal Holley 4412 2-barrel with choke horn

  - No XP carbs

- Carb spacer/adapter:

  - Maximum 1 inch, non-adjustable

  - No “super-sucker” style adapters

- No aftermarket fuel injectors or throttle bodies.

## 5. Rear Ends & Suspension

- Stock rear ends may use Moser axles, or a Ford 9”. Floaters allowed.

- Locked rear ends allowed; FWD cars may lock fronts.

- Rear-end mounts and suspension must remain in stock location.

- No wedge bolts (front or rear).

- No spring spacers or spring buckets.

- Stock trailing arms; no offset bushings.

- Trailing-arm mounts must be stock.

- Max 2¾ inches from bottom of rear-end housing to center of control-arm bolt.

- Stock upper & lower A-arms.

- Stock shocks only — no racing shocks.

- One shock per wheel in stock location.

- Shocks subject to a $400 claim for all four; track may claim at any time.

- Suspension must be fully stock and in stock location.

- Springs:

  - Stock-appearing only

  - Rear springs must be double-pigtailed and no taller than 14 inches

## 6. Wheels, Tires, & Brakes

- Stock wheels or 8-inch max racing wheels.

- Beadlocks allowed on right-side only.

- No homemade wheels.

- 1-inch lug nuts required on all four wheels (factory aluminum wheels may use stock nuts).

- All four tires must match in size — no stagger.

- DOT tires only (no racing tires).

- All four brakes must work.

- No brake bias systems.

## 7. Engine Claim Rule

- Engine may be claimed for $1750 plus exchange. Track keeps $250.

- Claim must be made at scales within 5 minutes of race completion.

- Driver must go directly to scales with claim money (cash).

- Eligibility to claim:

  - Must finish in top 5 on lead lap

  - Must have competed in 3 prior races

  - One claim per event

  - Must be same manufacturer (Chevy-to-Chevy, Ford-to-Ford, etc.)

- Items not included in the claim:
Carb, distributor, water pump & pulleys, water neck, motor mounts, fuel pump, fuel-pump rod & cover, exhaust, flywheel assembly.

- Failure to surrender a claimed engine results in forfeiture of all money and points for the event.