## TRANSPONDERS & RACE RECEIVERS (MANDATORY)

Any driver without a required transponder or race receiver during a heat or feature race is subject to immediate disqualification. No exceptions.

## TRANSPONDERS

- Drivers may rent a transponder from the track with:

  - A valid driver’s license

  - A $10 rental fee

- If a rented transponder is damaged while in the driver’s possession, the driver must pay the full replacement cost of $275 before any winnings or their driver’s license will be released.

- All rented transponders must be returned to the pit office immediately after the feature race to receive winnings and the driver’s license.

## RACE RECEIVERS

- Drivers must supply their own race receivers; no rentals are available.

- Replacement items sold at the pit office:

  - Earbuds: $3

  - Batteries: $1

# CAR REQUIREMENTS & SAFETY EQUIPMENT

### Safety Equipment

- 6-point roll cage: Mandatory

- 4-point racing harness: Mandatory

- Working fire extinguisher in every car: Mandatory

- Window nets: Highly recommended

- All glass must be removed

  - Windshield must be replaced with screen

  - Three (3) protective bars are required behind the screen

- Full interior removal required

### Driver Gear (Mandatory)

- Racing pants or long pants- No Exceptions!

- Racing jacket or long sleeves- No Exceptions!

- Full-face helmet- No Exceptions!

- Racing shoes or closed-toe shoes- No Exceptions!

### Vehicle Requirements

- Added weight must be painted white and clearly marked with car number.

- All doors must be chained, welded, or bolted shut.

- Tow chains or loop bars on front and rear — Mandatory.

- Car numbers on roof and both sides: 18" or larger

- Car must have a hood to start the race.

  - If the hood comes off during the race → Black flag and exit the track.

# ROLL CAGE RULES

- All classes must have a full 6-point roll cage. MANDATORY!

- Roll cages must be built from steel tubing only — no pipe allowed.

  - Minimum tubing: 1.5" OD × .095" wall thickness

- All tubing must be mandrel-bent and welded with ARC, MIG, or TIG.

- Cage must include:

  - One horizontal bar behind the driver, running left to right.

  - Minimum of 2 horizontal door bars on the passenger side.

  - Minimum of 4 horizontal door bars on the driver’s side.

  - One horizontal bar from left to right between the front cage posts at dash height.

- Additional bars or braces to protect the driver, radiator, etc. are allowed, but must remain inside the body lines.

  - Rub rails are allowed on the outside of doors as long as no edges protrude.

- If there is no frame, a mounting plate must be welded or bolted to the floor, and the cage tubing must be welded to the plate.

  - Plate specifications: ¼" thickness, 4" × 4" square.

  - If plates are bolted:

    - Must use at least 2 bolts per plate

    - Bolts must be ½" diameter minimum.

### Pit Conduct

- Pit speed limit: 5 MPH

- ATVS/UTVS/Golf Carts- Must abide by the speed limits. Children under  the age of 12 must be accompanied by an adult when operating.

- ATVS/UTVS/Golf Carts are not permitted to drive through the grandstands or behind the flag stand.

- Any ATV/UTV/Golf Cart without working lights is not permitted after dark.

- No fighting or harassment — Zero tolerance.

- All concerns must be directed to a track official only.

All concerns, disputes, or issues must be taken to a track official when one becomes available. Drivers/spectators are not permitted to enter the racing surface to engage with a track official. All concerns, disputes, or issues will be addressed after the last race of the night. In urgent cases an official may be radioed to come to the pits for a discussion.

All decisions of the track official, director, or owner will be FINAL.

*Reminder: The pit office staff do not have the ability to watch the race; therefore, they cannot handle race-related concerns, disputes, or issues.

### Race Procedures

- Heats: 7 Laps

- Features: 14 Laps.

- All classes will have a 25-minute time limit, unless additional laps are added for special events.

- All initial starts will be in double file formation. After laps have been officially scored if a caution occurs restarts will utilize Delaware Double-File formation.

- The pole sitter sets the pace.

- Only three (3) complete restarts are permitted.

After the third restart, the race will move to single-file formation.

- Cautions & Limits:

  - Multi-Car caution on the first lap: All Cars will retain their original position, and a complete restart will be granted.

  - Single-Car Caution will result in driver/car losing the position and going to the tail of the field regardless of what lap the caution was received on. This includes being charged with a jump.

  - Hitting another driver or car during a caution or after the race has concluded is strictly prohibited. This will result in immediate disqualification for the night, losing all earnings for the night.

  - Heats: Maximum of 2 cautions per driver

  - Feature: Maximum of 3 cautions per driver

  - After exceeding  the above limits driver will be instructed to leave track.

- Any driver change or car change must be reported to the pit building official.

  - Driver/Car must start in the tail of the heat or feature the change was required for.

  - Failure to report will result in disqualification.

- Drivers racing in more than one class must:

  - Pay $35 per class

  - Show proof of an armband for each class

- Only employees/track officials are permitted in the following: No Exceptions.

  - Infield

  - Pit building

  - Tower

  - Concession stands

### Disqualifications / Points

- If you are disqualified for motor, transmission, suspension violations, or for refusing a teardown or claim, you will lose all points earned for that night plus an additional 100 points.

- Any car that is determined to be dominant may be required to add weight at the discretion of track officials.

- Drivers must compete in at least ¾ of the completed races to qualify for track points.

- Points stay with the driver of each class and do not transfer to another class.

- Top 5 drivers in each regular season class will receive awards at the end of the race season. All requirements stated above must be met.

- Drivers will receive points for the following:

- Show-up points

- Heat finishing position

- Feature finishing position

# 2026 PROTEST RULES — FACTORY STOCK & BOMBER

(Refer to individual class rulebooks for additional class-specific protest/claim procedures.)

### Eligibility to Protest

A driver must:

- Finish in the top 10 and pass tech (be legal)

- Be on the lead lap

- Have competed in at least three prior races

### Claim Procedures

- A claim must be made within 5 minutes after the race at the scales, directly to the tech inspector.

- Leaving the track and returning is not permitted when making a claim.

### Protest Fees

- Complete Tear-Down: $500

  - Track retains: $100

- Top-End Protest: $250

  - Track retains: $50