## GENERAL WEIGHT RULE

- Minimum weight: 3200 lbs. with driver

# ENGINE RULES

## Engine Setback (Both Options)

- No engine setbacks. Must be in stock location.

## ENGINE OPTION 1 — GM 602 SEALED CRATE

- Must be a GM 602 sealed crate engine.

- Any altering of bolts, seals, or anything suspicious results in disqualification until proven legal.

## ENGINE OPTION 2 — OEM STYLE SMALL BLOCK

### Displacement Limits

- Chevrolet: 362 cu in max

- Dodge: 368 cu in max

- Ford: 363 cu in max

### Intake

- Any cast iron or aluminum dual-plane intake allowed.

- Intake may be drilled for Vortec heads.

### Cylinder Heads

- Must be stock cast iron heads.

- OEM Vortec or Vortec-style allowed.

- Maximum valve sizes:

  - Intake: 1.94"

  - Exhaust: 1.50"

- No porting or polishing.

### Rocker Arms & Valvetrain

- Stock-type stamped steel rocker arms only.

- Screw-in studs and guide plates allowed.

- No roller rockers or roller tips.

- No roller, mushroom, or ceramic lifters.

- Any hydraulic or solid cam allowed up to .480" max valve lift.

- Stock-style lifters only.

- No gear or belt drives—must use chain timing components.

### Block, Pistons, and Rotating Assembly

- Stock bore (+.060) and stroke only.

- All block casting numbers must be visible.

- Flat-top pistons only (3-ring groove).

  - No dome pistons.

- Stock-length I-beam rods only.

  - No H-beam rods.

- Factory or forged cast-iron/steel crankshaft only.

  - Minimum weight: 50 lbs

  - No knife-edging.

- Maximum compression: 180 psi.

# EXHAUST

- Headers or stock manifolds are allowed.

- No crossover, 180-degree, or Tri-Y headers.

# IGNITION

- HEI-style ignition only.

- No MSD boxes of any type.

- No external coils or external-type distributors.

- MSD-brand HEI-style distributor allowed.

- Rev limiter allowed.

# CARBURETOR

- Holley 4412 only on shop motors— must pass gauge checks.

- Crate motors may run a 4-barrel 650 CFM.

- Choke horn required.

- Spacer/adapter may not exceed 1 inch.

- No removable or modified air bleeds.

- Fuel: Gasoline only (110 octane max).

  - No E85 or C85.

# TRANSMISSION

### Manual

- OEM/stock-type only.

- Minimum 10.5-inch clutch.

- Steel flywheel only.

### Automatic

- Must use working 11-inch minimum torque converter.

- No aftermarket racing transmissions (Bert, Brinn, Falcon, Jerico).

# SUSPENSION

- Suspension must remain in stock location.

### Front Suspension

- Non-adjustable tubular upper control arms allowed.

- Lower control arms must be stock.

- Stock ball joints only for the car model—no longer/shorter versions, no shims.

- Front spring cups are allowed (non-adjustable).

### Springs

- Aftermarket steel springs are allowed.

- No fiberglass or composite leaf springs.

- No progressive-rate coil springs.

- Lowering blocks allowed but non-adjustable only.

### Shocks

- Aftermarket steel shocks allowed.

- No rebuildable shocks.

- Shock spacing may be changed.

### Rear Suspension

- No adjustable bars or weight jacks.

- No rear spring buckets—spring pocket must be flush with frame.

- Stock rear ends may be locked.

- Ford 9" or floater rear ends allowed.

- Rubber bushings required front & rear.

### Brakes

- All 4 brakes must work.

- Rear disc brakes allowed (steel rotors & hats, stock cast calipers).

- Aftermarket pedals are allowed.

- No brake adjuster reachable by driver while seated.

### Other

- No traction control devices.

- Driveshaft must be steel, painted white, with safety strap.

# BODY RULES

- Any 1960+ American-made sedan with stock frame.

- Minimum wheelbase: 101 inches (no shortening or lengthening).

- Body must be stock appearing for make of car.

- Body may be steel or aluminum.

### Floor & Firewall

- May be stock or fabricated using same-thickness steel.

- Must extend past driver seat to front of rear-end yoke.

- Must extend from driver-side frame rail to passenger-side frame rail.

- Both sides must be at same height.

### Roof

- Must be stock appearing.

- Must be within 2.5 inches front-to-back and side-to-side of OEM shape.

- Fiberglass replacement roof allowed (e.g., Speedway 910-54177).

### Bumpers & Body Features

- Front and rear bumpers in stock location (tubular allowed with tow hook).

- Plastic nose allowed (no wedge nose).

- Fiberglass hoods allowed.

- Cockpit may be enclosed (must include inspection hole).

- Tail of car must be completely enclosed.

# TIRES & WHEELS

- 8-inch maximum wheel width

- Allowed tires:

  - H500 Only.

- Grooving and siping allowed.